Experiment 1- Linear vs Cubic Motion Interpolation

Date: 1/19/2026

Objective:
 Compare the motion profiles of a single joint moving from 0° → 90° using linear interpolation versus cubic (S-curve) interpolation. Analyze overshoot, settling time, and final error to determine which method provides smoother motion.

Method:

- Simulated a single joint in Python using numpy and matplotlib.

- Linear interpolation: θ(t)=θstart​+(θtarget​−θstart​)×Tt​

- Cubic (S-curve) interpolation: θ(t)=θstart​+(θtarget​−θstart​)×(3s2−2s3),s=Tt​

- Calculated metrics:

  - Overshoot

  - Settling time (first time staying within ±1° of target)

  - Final error

- Plotted both motion profiles on the same graph.

- Saved plot to results/plots/joint_motion_comparison.png and metrics to results/data/metrics.txt.

Observations:

- Both linear and cubic interpolation reach the target angle.

- Cubic motion is smoother, reducing abrupt changes at start and end.

- Overshoot is slightly lower with cubic motion (though minimal in this setup).

- Settling time is effectively the same due to moderate total time (2s).

Next Steps:

- Perform a parameter sweep testing multiple total times (1, 2, 3 seconds) and step counts (50, 100, 200) to see the effect on overshoot, settling time, and smoothness.

- Compare trends for linear vs cubic motion and save metrics for analysis.

Experiment 2- Parameter Sweep: Total Time & Steps

Date: 1/19/2026

Objective: Analyze how different total times (1, 2, 3s) and number of steps (50, 100, 200) affect linear vs cubic motion interpolation.

Method:

- Swept total_time and num_steps

- Calculated overshoot, settling time, final error

- Saved metrics in CSV: results/data/metrics_sweep.csv

- Saved plots for each combination in results/plots/

Observations:

- Cubic motion consistently smoother

- Higher step counts produce smoother curves

Next steps:

- Generate comparison graphs (linear vs cubic) showing trends

- Summarize findings for resume / presentation

- Add PID control for the single joint

Experiment 3- PID Control of a Single Joint

Date 1/20/2026

Objective: Test the use of a PID controller to move a single joint from a start angle to a target angle, analyzing overshoot, settling time, and steady-state error.

Setup:

- Joint starts at 0 degrees, target = 90 degrees

- Simulation time: 2 seconds, 100 steps

- PID controller gains: Kp, Ki, Kd, are all input based in PID python script

- Metrics collected:

  - Overshoot (max angle beyond target)

  - Settling time (time to stay within ±1 degree of target)

  - Final error (difference between final angle and target)

Method:

- Implemented PID control in Python with simple Euler integration:

- Theta updated each timestop using theta[i] = theta [i-1] + control x dt

- Metrics automatically calculated and saved to metrics_pid.csv

- Plot generated showing joint angle over time with PID response and target line

Reminders:

Kp = Proportional: pushes the joint toward the target based on current error.

Ki = Integral: accumulates past error to eliminate steady-state offset.

Kd = Derivative: dampens motion, reduces overshoot and oscillations

Kp too low —> slow response

Kp too high —> overshoot/oscillation

Ki too low —> may not reach target exactly

Kd too low —> can overshoot

Kd too high —-> too sluggish

Results:

- PID successfully moved the joint toward the target

Experiment 4- Optimal PID tuning (find best Kp, Ki, Kd)

Date: 1/20/2026

Objective: Determine the optimal PID gains (Kp, Ki, Kd) that achieve the fastest, most accurate, and smoothest motion.

Setup:

- Simulation 0 degrees to 90 degrees, total time = 2s, 100 steps

- Metrics: overshoot, settling time, final error

- Gain ranges:

  - Kp: 4, 5, 6

  - Ki: 0.5, 1, 1.5

  - Kd: 0.3, 0.5, 0.7

Methoud:

- Optimal PID search in script optimal_pid.py

- Record metrics to CSV

- Cost function: cost = settling time + overshoot + abs(final_error)

- Identify combination with lowest cost —> “optimal gains”

Results:

- Optimal gains: Kp= 7, Ki = 1.0, Kd = 0.5

- Achieves fast rise, minimal overshoot, near zero final error

- Plot saved as pid_optimal.png

Conclusion:

- Systematic tuning shows the impact of each gain on motion performance

- Demonstrates research-style optimization approach

Experiment 5: Comparison of Motion Methods

Objectives: Compare Linear, Cubic, and optimized PID motion to visualize differences in speed, accuracy, and smoothness.

Date: 1/19/2026

Setup:

- Use results from experiment 1 (Linear/Cubic) and Experiment 3 (Optimal PID)

- Metrics: overshoot, settling time, final error

- Simulation: 0 degrees —> 90 degrees, total_time = 2s, steps = 100

Method:

- Reconstruct trajectories

- Plot Linear, Cubic, and PID on the same figure with target line

- Annotate metrics for each method

- Save comparison plot: comparison_linear_cubic_pid.png

Results:

- Linear: fast, predictable, no overshoot

- Cubic: smooth, slightly slower

- PID: fastest settling, minimal overshoot, very accurate

- Clear visual shows PID superiority with feedback control

Conclusion:

- Optimized PID outperforms open-loop methods in speed and accuracy

- Demonstrates importance of feedback in motion control